MyDesign Portfolio

Element D Initial Template

# How to use this template

Teachers: You may customize this document before providing it to students.

Students: The template below is one way to present your design ideas, but the specific assignment given by your instructor may require a different structure or dictate different details. If you make a copy of this template to edit for your own design ideas, be sure to delete the instructions.

# Guidance before filling out the template on the next page:

Because you will iterate on your design multiple times, you will need to create multiple copies of these sections. Think of Element D as a diary that you will date and keep adding to throughout your work until your final iteration. It’s critical that iteration shines through in this Element.

You should begin with many design ideas that reflect the individual ideas of your group members and then the group ideas. Number these ideas and include a date, description, drawing, and details for each.

After you have created an initial set of ideas, document your decision making on which design to include, ideally with a design matrix. Create a plan of work, ideally using a Gantt chart that may be housed in this document or in a different day-to-day document. Generate a design blueprint with enough details that someone else not familiar with your project could also build your design.

You will then proceed forward to build and test this design, returning to Element D afterwards when you are ready to create more design ideas. Fill out each of these steps again.

# Element D:  Design Concept Generation, Analysis, and Selection

Design Idea #1 - Box

Date: October 24, 2023

Description: A fully enclosed Plywood Acoustic Chamber lined with egg-carton foam. It encapsulates the entire printer to block airborne fan and motor noise.

Drawing that includes distinguishing information such as scale, dimensions, and materials: [Drawing: 60x60x60cm cube. Materials: 12mm MDF panels, 25mm Acoustic Foam lining, Acrylic front window (30x30cm), and rubber door seals.]

Design Idea #2 - Vibration Dampening Feet

Date: October 25, 2023

Description: Specialized TPU Spring-Dampers that replace the stock printer feet. This focuses on reducing structural vibration and "desk hum" rather than fan noise.

Drawing that includes distinguishing information such as scale, dimensions, and materials: [Drawing: 40mm diameter, 30mm height. Materials: 95A Durometer TPU (3D Printed) with internal honeycomb infill (20%) for energy absorption.]

Design Idea #3 - SQUASH Ball Gantry Mounts

Date: October 26, 2023

Description: A Hybrid Isolation Base consisting of two wooden plates sandwiching four standard squash balls. This decouples the printer from the surface to eliminate low-frequency resonance.

Drawing that includes distinguishing information such as scale, dimensions, and materials: [Drawing: Two 50x50cm Baltic Birch plywood sheets (18mm). Materials: 4x Yellow Dot Squash Balls seated in 35mm recessed circular pockets.]

Design Idea Comparison and Selection (D1, D2)

When you’ve finished generating ideas, you must compare your design options according to all your design requirements. Incorporate the priority of those requirements when comparing design options. A decision matrix (perhaps a Pugh Matrix) is encouraged since it allows for systematic, weighted comparisons. If using weights in a decision matrix, be sure to explain and justify the weighting based on your design requirements. You may use the weighted design matrix template below, inserting your criteria and design options.

Weighting Justification: "Noise Reduction" is weighted highest (0.4) as it is the primary goal. "Ease of Mfg" (0.2) and "Cost" (0.2) ensure project feasibility. "Heat Management" (0.1) and "Footprint" (0.1) ensure printer safety and usability.

|  | weight | Option 1 (Box) | Option 2 (Feet) | Option 3 (Base) | Option 4 |
| --- | --- | --- | --- | --- | --- |
| Criterion A: Noise Reduction | 0.4 | 5 | 2 | 3 |  |
| Criterion B: Ease of Mfg. | 0.2 | 2 | 5 | 4 |  |
| Criterion C: Cost | 0.2 | 1 | 5 | 3 |  |
| Criterion D: Heat Mgmt. | 0.1 | 1 | 5 | 5 |  |
| Criterion E: Footprint | 0.1 | 2 | 5 | 4 |  |
| Weighted Total | 1.0 | 2.9 | 3.8 | 3.5 |  |

Final Selection: Design Idea #2 (Vibration Feet) selected for its high score in cost-effectiveness, heat management (no enclosure heat creep), and minimal desk space usage.

Design Blueprint (D4)

Describe all of your design's details such that someone unfamiliar with your project could build your prototype. Include sketches, dimensions, CAD (if taught in your class), manufacturing methods, material choices, supply lists, and costs.

Design Blueprint #1

Date: October 28, 2023

Sketch: [Detailed 3D view of a cylindrical foot with a hollow center and 20% Gyroid infill pattern. Top has an M4 bolt hole for printer attachment. Base has a 2mm non-slip rubber pad.]

Manufacturing Methods:

Step 1: Slice the provided STL file using a 0.2mm layer height and "Gyroid" infill pattern at 20% density.

Step 2: Print using TPU (Flexible Filament) on a direct-drive 3D printer with a bed temperature of 50°C.

Step 3: Remove supports and bolt the feet into the existing T-nuts on the 3D printer's bottom aluminum extrusion frame.

Supply List and Costs:

| Item | Cost each | # needed | Total |

| :--- | :--- | :--- | :--- |

| TPU Filament (1kg Spool) | $28.00 | 0.1 | $2.80 |

| M4 x 10mm Bolts | $0.25 | 4 | $1.00 |

| M4 T-Nuts | $0.30 | 4 | $1.20 |

| Adhesive Rubber Pads | $5.00 | 0.2 | $1.00 |

| | | | |

| | | | |

| | | | |

| | | | |

| Total Cost | | | $6.00 |